RELIEF VALVES

LIQUID EXPANSION

NOTATION:

r 	= Density of liquid (kg/m3)

C 	= Specific heat of liquid (J/kg °C)

H 	= Heat flux into the system (W)

a 	= Thermal expansion coefficient of the liquid (1/°C)

t 	= Time (s)

V	= Volume (m3)

M	= Mass flow rate (kg/s)

m	= Mass flow rate (kg/h)

T	= Temperature (°C)

Ps	= Set pressure (kPag)

OP= Overpressure

Pb	= Back pressure (kPag)

Kd=Coefficient of discharge

A	= Theoretical area (m2)

A*= Required area by AS1271 (mm2)

CALCULATIONS:

The relieving flow rate is equal to the rate of expansion of the volume	dV

dt

rate of rise of volume

rate of rise of temperature

increase in volume as function of temperature

Hence:

volumetric flow rate

M =  r (dV/dt) =  a. H/C	mass flow rate (kg/s)

m =  3600 r (dV/dt) =3600 a. H/C	mass flow rate (kg/h)

DP = 1000 . ((1+OP)Ps-Pb)	Pressure drop in Pa

_______

M = A Ö 2 r DP              Theoretical discharge of an orifice

a  H

A = ____________        Theoretical area required (m2)

C ( 2 r DP )½

22360  a H

A* = _____________________           Required area (mm2)	H in Watts

0.9 Kd C [r ((1+OP)Ps - Pb)]½	Ps in kPag

Pb in kPag

r in kg/m3

C in J/kg°C

If   Pb is 0 (relieving directly to atmosphere) and the allowed overpressure is OP=25%, and if we use the specific gravity ,G, of the liquid, we get a further simplification:

632.44  a H

A* = _____________________      Required area (mm2)	H in Watts

0.9 Kd C  [G . Ps ]½	Ps in kPag

Pb in kPag

C in J/kg°C